Calcium Chloride

General Characteristics:

Formula:	CaCl2(anhydrous); CaCl2·2H2O(dihydrate)

Molecular weight:	110.98(anhydrous); 147.01(dihydrate)

Calcium chloride occurs as white, hard fragments, granules,

Appearance:	or powder.

Odor:	Odorless, slight bitterness.

CAS Number:	anhydrous [10043-52-4]; dihydrate [10035-04-8]

EINECS No.:	233-140-8

INS:	509

Solubility:	In water -Soluble; In alcohol -soluble.

Uses:

- Nutrient supplement、Stabilizer and coagulant.

- Thickener、 Processing agent、Tissue improver.

- Chelant、Calcium intensifier、Refrigerant for freezing.

- Desiccant and anticaking agent 、Inhibitory microbiological agent .

Packing and storage:

- 25kg net paper bag and PE bags sealed inside.

- Store in a well-closed bag at room temperature, protect from light, moisture and pest infestation.

- Shelf life--- two years

GMO-Status:

The product is a non GMO product and is free from any recombinant DNA.

Irradiation/Radioactivity:

Yunbo’s Calcium chloride was never subjected to any kind of ionized irradiation and contains no radioactivity not even in minor amounts.

BSE/TSE:

No raw materials from bovine origin are used nor are any bovine constituents present in the product.

Specifications:(FCC/E509/USP/EP)

Calcium Chloride(Anhydrous)

| Test Parameter | FCC | E509 | USP-NF |
| --- | --- | --- | --- |
| Assay【CaCl2】% | 93.0~100.5% | ≥93.0% | ≥99.0% |
| Test for calcium | Passes test | Passes test | Passes test |
| Test for chloride | Passes test | Passes test | Passes test |
| pH | / | / | 4.5-9.2 |
| Fluoride | ≤ 0.004%. | ≤ 0.004%. | / |
| Arsenic | ≤3.0ppm | ≤3.0ppm | / |
| Lead | ≤5.0ppm | ≤2.0ppm | / |
| Mercury | / | ≤1.0ppm | / |
| Heavy metals（Pb） | / | / | ≤ 0.001%. |
| Acid-insoluble matter | ≤0.02% | / | / |
| Magnesium and Alkali Salts | ≤5.0% | ≤5.0% | ≤1.0% |

Calcium Chloride(Dihydrate)

| Test Parameter | FCC | E509 | USP-NF | Ph.Eur |
| --- | --- | --- | --- | --- |
| Assay【CaCl2·2H2O】% | 99.0%~107.0% | / | 99.0%~107.0% | 97.0%~103.0% |
| Test for calcium | Passes test | Passes test | Passes test | Passes test |
| Test for chloride | Passes test | Passes test | Passes test | Passes test |
| pH | / | / | 4.5-9.2 | / |
| Fluoride | ≤ 0.004%. | ≤ 0.004%. | / | / |
| Sulfates | / | / | / | ≤ 300.0ppm |
| Arsenic | ≤3.0ppm | ≤3.0ppm | / | / |
| Lead | ≤5.0ppm | ≤2.0ppm | / | / |
| Mercury | / | ≤1.0ppm | / | / |
| Aluminium | / | / | / | ≤1.0ppm |
| Iron | / | / | / | ≤10.0ppm |
| Barium | / | / | / | Passes test |
| Heavy metals（Pb） | / | / | ≤ 0.001%. | / |
| Acid-insoluble matter | ≤0.02% | / | / |  |
| Magnesium and Alkali | ≤4.0%. | ≤5.0% | ≤1.0% | ≤0.5% |